# SOUND

Steps to power on the sound board, speakers and sub (woofer).

IN THIS ORDER…

- Turn on sub woofer that sits on the stage on the back of the box.  You will not hear anything as the board will not have been turned on yet.

- Flip the POWER button on in the tech booth.  This will power on the board, sub and speakers one at a time so that no pops or noise are heard.

- On the sound board, push the small white View Scene button and on the LED Screen select MASTER *** (now the only scene on there). Click load and confirm with the arrow keys. This scene has been eq’d by a professional so please do not make ANY changes without talking to Sabrina.

- On the stage in the rack, push the 2 soft buttons on the In Ear Monitor component.

FYI – piano and drums are now on 2 channels each (left and right).

There are now 3 DCA’s – Drums, Band (all instruments without drums: piano/bass/ac gtr/egtr/synth) and Vocals. You can now mix with only these 3 faders on the right side of the board (channels are red).

PLEASE VIEW THE VIDEO ON DCA’S FOR MORE DETAILS.

POWER OFF – In this order…

- Turn off sub on stage.

- On board LED screen choose set up button (like usual) and hit shutdown button and confirm with the arrow keys.

- Turn off Power Switch – everything will power off in sequence to prevent pops.

- Place cover back on board.

- Remove all personal effects and clean up area.

Thank you !!!